# List of keywords in the settings files

The keywords are all read and input in the module named Configure.m.

# Geometry Block

Starts with

#-------------------------------------------------

# GEOMETRY

#-------------------------------------------------

## LENGTH

Length of the core + unit

Example: LENGTH	15	cm

## DIAMETER

Diameter of the core + unit

Example: DIAMETER	7.5	cm

# Rock Block

Starts with:

#-------------------------------------------------

# ROCK

#-------------------------------------------------

## POROSITY

Porosity of the core in fraction

Example: POROSITY	0.297

## PERMEABILITY

Permeability of the core + unit

Example: PERMEABILITY	260	mD

## INITIALWATER

Initial water saturation in fraction

Example: INITIALWATER	1

# FLUID Block

Starts with

#-------------------------------------------------

# FLUID

#-------------------------------------------------

## DENSITYW

Wetting phase density + unit

Example: DENSITYW	1000	kg/m^3

## DENSITYNW

None-wetting phase density + unit

Example: DENSITYNW	800	kg/m^3

## VISCOSITYW

Wetting phase viscosity + unit

Example: VISCOSITYW	1	cP

## VISCOSITYNW

None-wetting phase viscosity + unit

Example: VISCOSITYNW	5	cP

# Process Block

Starts with:

#-------------------------------------------------

# PROCESS

#-------------------------------------------------

Type of the experiment + process

Types: SS, USS, CENT

Processes: Drainage, Imbibition

Example: SS	Imbibition

# Block Simulation

Starts with:

#-------------------------------------------------

# SIMULATION

#-------------------------------------------------

Defines is one is doing a forward or a history match simulation

Example: TYPE		Forward

Example: TYPE		HistoryMatch

The number of cells for the core

Example: NCELLS		80

The number of boundary cells at each end of the core and the initial water saturation at these boundary cells

#         number    first_cell_Sw   last_cell_Sw

BCELLS		1           0               0

Maximum time step allowed for the simulation + unit

MAXTIMESTEP 	2		hour

Number of geometric breaks down of the time steps at the beginning of each schedule row

RAMPUPSTEPS      10

If used, the maximum saturation change for the solver is set to 5%, which increases the accuracy but reduces simulation speed, useful for e.g. USS experiments with pc=0

HIGH_PERCISION_MODE

# Schedule Block

Starts with:

#-------------------------------------------------

# SCHEDULE

#-------------------------------------------------

Directory of the schedule file to input

#			fileName

FILENAME	.\examples\Data\SS\Imbibition\sched_case1.txt

Initial pressure of the whole core

PINI		1		bar

Initial pressure at the right side of the core (x=core length)

POUT		1		bar

Start up time in case of a centrifuge experiment: time + unit + rpm + unit

STARTUP     100     second      2325    rpm

Distance between the centre of the centrifuge rotation axis and the centre of the core

CENTRAD     14.89   cm

# Observation Block

Starts with

#-------------------------------------------------

# OBSERVATION

#-------------------------------------------------

Input the experimental observations to compare with forward simulation results and history match

Possible keywords: PRESSURE, PRODUCTION, SWAVG, SATPROFILE + file directory

#			fileName

PRESSURE	.\examples\Data\SS\Imbibition\pressure_obs_case1.txt

SWAVG       .\examples\Data\SS\Imbibition\swavg_obs_case1.txt

SATPROFILE  .\examples\Data\SS\Imbibition\SatProfile_obs_case1.xlsx

# Saturation function Block

Starts with

#-------------------------------------------------

# SATURATION FUNCTIONS

#-------------------------------------------------

KR keyword to input relative permeability saturation function + model name (e.g. MODIFIED-COREY/TABLE + parameters in order and tab separated/ directory to the table for the list and order of the parameters have a look at the “. \ad-scal\models\SaturationFunctions\Forward\Kr\read_kr_parameters_from_settings.m”

#KR         MODIFIED-COREY      	Swc     Sor     krw@Sor     kro@Swc     nW      nNW

KR          MODIFIED-COREY      	0.2    0.2     0.5         0.5      3       3

Similar to the previous, use PC keyword for capillary pressure. For the list and order of the parameters have a look at “. \ad-scal\models\SaturationFunctions\Forward\Pc\read_pc_parameters_from_settings.m”

PC			TABLE        .\examples\Data\SS\Imbibition\pc_smooth.txt

# Plot Options block

Starts with:

#-------------------------------------------------

# PLOT OPTIONS

#-------------------------------------------------

Style can be docked or normal

STYLE			docked

Colormap can be jet, parula, hsv

COLORMAP		jet

For possible units have a look at the available units list

DISPLAYTIME		hour

DISPLAYLENGTH		cm

DISPLAYVOLUME		cm^3

DISPLAYPRESS		bar

DISPLAYRATE		cm^3/hour

# Output options block

Starts with

#-------------------------------------------------

# OUTPUT OPTIONS

#-------------------------------------------------

Define the desired output options and the directory to output them

SATPROFILE		.\Results\satProfile.xlsx

Saves a settings file which can be re-used for setting up the simulation

SAVECONFIG		.\Results\config.txt

QUANTITIES		.\Results\output.txt

If you use each of the key words here this is means the output is generated for the corresponding property - !make sure to activate this option before activating the parameters below.

TIME

Average water saturation

SWAVG

Injection and production rates

INJ

PROD

DELTAP

# Objective function configurations block

Starts with

#-------------------------------------------------

# OBJECTIVE FUNCTION CONFIGURATIONS

#-------------------------------------------------

Settings the configurations for history match simulations

The excel directory to input initial point, and boundaries for KR/PC parameters to iterate upon

EXCEL_FILE_NAME    kr_modified_corey.xlsx

EXCEL_FILE_PATH    .\App\Input_templates_for_SCAL_history_match

If activated, the history matching is started from n random points within the boundary (defined in the random number keyword below) points, to increase the chance of finding the global minimum

MULTIPOINT

MULTIPOINT_RANDOM_NUMBERS      10

If activated, the relative permeability is history matched

KR

Define the model of the relative permeability for history match, it has to match the parameters in the excel template, you can also use POINT-BY-POINT

KR_MODEL    MODIFIED-COREY

If activated, the capillary pressure is history matched

PC

Define the model of the capillary pressure for history match, it has to match the parameters in the excel template, you can also use POINT-BY-POINT

PC_MODEL    POINT-BY-POINT

Define the weights of each measurement in the objective function calculations

PDIFF_WEIGHT    1

SWAVG_WEIGHT    1

PROD_WEIGHT     1

SAT_PROFILE_WEIGHT      1

Define the error of measurements, for normal history matching leave them as 0, for MCMC simulations use the relative error in percentages

PDIF_ERROR      0

SWAVG_ERROR     0

PROD_ERROR      0

SAT_PROFILE_ERROR       0

# History match configurations block

Starts with:

#-------------------------------------------------

# HISTORY MATCH CONFIGURATIONS

#-------------------------------------------------

If activated uses parallel CPU computation

USE_PARALLEL

Parameters related to the fmincon function of MATLAB

OPTIMALITY_TOLERANCE        1e-6

STEP_TOLERANCE          1e-3

MAX_FUNCTION_EVALUATIONS    6000

SCALE_PROBLEM

Can use normal here for single history of 1 experiment and Simultaneous to history match 2 experiments at the same time e.g. SS + CENT

OBJECTIVE_FUNCTION_TYPE    normal

Directory of the second experiment setting file to history match

CENT_FILE_NAME      ABC.txt

CENT_FILE_PATH      C:\Users\Omidreza\Documents\GitHub\SCAL

History match algorithm for fmincon (e.g. active-set, interior-point) or can use ga-multi-obj for multi objective optimization with genetic algorithm

HISTORYMATCH_ALGORITHM      active-set

# MCMC configurations block

Starts with

#-------------------------------------------------

# MCMC SPECIFIC CONFIGURATIONS

#-------------------------------------------------

Here we set the settings for monte carlo (MCMC) simulations with paramonte package

RANDOM_SEED     3751

SAMPLE_REFINEMENT_COUNT     1

CHAINSIZE       5000

#MPI_ENABLED